SmartHUB Tech & Internet Café

Network Infrastructure Requirements Document

| Document ref. | SMHUB-NET-REQ-001 | Status | Final – v1.0 |
| --- | --- | --- | --- |
| Prepared by | IT Infrastructure Team | Date issued | May 2026 |
| Approved by | General Manager | Review date | November 2026 |

# 1. Executive Summary

SmartHUB Tech & Internet Café is a modern multi-zone digital facility located in Cape Town, South Africa. The facility provides public internet access, a dedicated gaming lounge, internet café spaces, and internal staff operations - all served by a single converged network infrastructure.

This document defines the complete network infrastructure requirements for the SmartHUB facility. It covers logical and physical design, VLAN segmentation, security policies, device specifications, and project delivery milestones. The network must enforce strict guest isolation, and provide a secure and manageable operational backbone for staff.

All designs are intended to be implemented and demonstrated in Cisco Packet Tracer 8.x as a full-scale simulation, suitable for professional portfolio and technical documentation purposes.

# 2. Business Context & Objectives

## 2.1 Facility profile

| Attribute | Detail |
| --- | --- |
| Business name | SmartHUB Tech & Internet Café |
| Location | Cape Town, Western Cape, South Africa |
| Facility size | Single-floor premises – approx. 250 m² |
| Operating hours | 07:00 – 22:00, seven days a week |
| Expected users | Up to 60 concurrent users across all zones |
| Network type | Converged wired and wireless LAN with internet uplink |
| Simulation tool | Cisco Packet Tracer 8.x (full topology) |

## 2.2 Business objectives

- Provide reliable, fast public internet access to walk-in customers and internet café customers.

- Deliver a low-latency, high-bandwidth experience for the gaming lounge.

- Always protect internal staff systems and servers from public network access.

- Enable centralized billing and session management through a captive portal system.

- Ensure all network devices are securely managed via SSH from a dedicated management VLAN.

- Comply with best-practice network segmentation to protect customer data and business assets.

# 3. Scope of Work

The following items are within scope for this infrastructure project:

- Full Layer 2 and Layer 3 network design including VLAN segmentation and inter-VLAN routing.

- DHCP and DNS service provisioning for all zones.

- Internet access via NAT/PAT for all customer-facing zones.

- Captive portal web server for guest authentication and session management.

- Access control lists (ACLs) enforcing inter-zone security policies.

- Wireless access points with zone-specific SSIDs.

- Secure device management via SSH and a dedicated management VLAN.

- Spanning Tree Protocol (STP) configuration for loop prevention.

- Full simulation, verification, and documentation in Cisco Packet Tracer.

The following items are out of scope for this phase:

- Physical hardware procurement and rack installation.

- Wide Area Network (WAN) redundancy or ISP failover.

- Cloud-hosted services or SD-WAN integration.

- Advanced wireless controller (WLC) configuration.

# 4. Network Zones & VLAN Plan

The network is divided into six distinct zones; each assigned a dedicated VLAN for traffic isolation and policy enforcement. All VLANs are trunked between the distribution Layer 3 switch and core edge router using IEEE 802.1Q encapsulation.

| Zone / VLAN name | ID | Subnet | Purpose | Default gateway | Access level |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 – Public browsing | 10 | 192.168.10.0/24 | Walk-in customers, browsing stations | 192.168.10.1 | Internet only |
| VLAN 20 – Gaming lounge | 20 | 192.168.20.0/24 | Dedicated gaming PCs and consoles | 192.168.20.1 | Internet only |
| VLAN 30 – Internet Café | 30 | 192.168.30.0/24 | Café customers needing print and file access | 192.168.30.1 | Internet + print server |
| VLAN 40 – Staff / admin | 40 | 192.168.40.0/24 | Cashiers, managers, back-office PCs | 192.168.40.1 | Full internal access |
| VLAN 50 – Servers / DMZ | 50 | 192.168.50.0/24 | Captive portal, billing, DNS, CCTV NVR | 192.168.50.1 | Restricted (ACL-controlled) |
| VLAN 99 – Management | 99 | 10.0.99.0/24 | Out-of-band switch and router management | 10.0.99.1 | SSH from VLAN 40 Network Admin PC only |

Note: All guest-facing VLANs (10 and 20) are denied access to any RFC 1918 address range other than the designated DNS server. Inter-VLAN routing is performed by the dual MLS distribution switches via SVIs. The core router handles internet uplink and NAT/PAT only.

# 5. IP Addressing Scheme

## 5.1 DHCP pool summary

| VLAN | DHCP range | Excluded (static) | Lease time | Notes |
| --- | --- | --- | --- | --- |
| VLAN 10 | 192.168.10.10 – .200 | 192.168.10.1 – .9 | 2 hours | Short lease – walk-in customers |
| VLAN 20 | 192.168.20.10 – .100 | 192.168.20.1 – .9 | 4 hours | Gaming sessions; fixed pool size |
| VLAN 30 | 192.168.30.10 – .80 | 192.168.30.1 – .9 | 8 hours | Internet Café members; longer lease |
| VLAN 40 | Static only | 192.168.40.1 – .20 | N/A | All staff PCs assigned static IPs |
| VLAN 50 | Static only | 192.168.50.1 – .20 | N/A | All servers have fixed assignments |
| VLAN 99 | Static only | 10.0.99.1 – .20 | N/A | Switch SVIs and router sub-interface |

## 5.2 WAN / ISP link

- WAN interface IP: 203.0.113.2 / 30 (simulated public range)

- WAN interface IP: 203.0.113.6 / 30

- Default route: Ip route 0.0.0.0 0.0.0.0 203.0.113.1

- NAT pool: Overload using WAN interface IP (PAT)

# 6. Device Inventory

The following devices form the simulated topology in Cisco Packet Tracer. All Cisco IOS devices must run an IOS version that supports the technologies listed in Section 8.

| Device | Qty | Role | Notes |
| --- | --- | --- | --- |
| Cisco 2911 Router | 1 | Core / edge router | NAT/PAT, default route to ISP, DHCP relay, WAN-facing ACLs |
| Cisco 3650 Switch | 2 | Distribution MLS (MLS-1 & MLS-2) | SVIs per VLAN, HSRP per VLAN, EtherChannel inter-MLS trunk, STP root/secondary |
| Cisco 2960 Switch | 5 | Access switches | One per zone (incl. dedicated server switch SW-5) – dual uplinks to both MLS devices, VLANs, port security, STP |
| Generic WAP | 4 | Wireless APs | SSIDs: SmartHUB-Public, SmartHUB-Gaming, SmartHUB-Cowork |
| PT Generic Server | 7 | Server farm | DHCP, DNS, HTTP (captive portal), FTP (file share), CCTV/NVR |
| PCs / Laptops / Tablets | 14+ | End devices | Distributed across all zones for connectivity verification |
| IP Phone (optional) | 2 | VoIP | Staff desk phones on VLAN 40 – voice VLAN extension |

# 9. Zone Functional Requirements

## 9.1 Public browsing zone (VLAN 10)

- Customers must receive an IP address automatically via DHCP upon connecting.

- All DNS queries must be resolved by the internal DNS server (192.168.50.11).

- First HTTP request must redirect customers to the captive portal landing page.

- Internet access is permitted via NAT/PAT after portal acceptance.

- No access to any other internal VLAN or server except DHCP, DNS, and WEB servers is permitted.

- Session lease time must not exceed 2 hours without re-authentication.

## 9.2 Gaming lounge zone (VLAN 20)

- Dedicated DHCP scope sized for a maximum of 20 concurrent gaming stations.

- No inter-VLAN communication is permitted – complete isolation from all zones.

- Port security must be enabled on all access switch ports with a sticky MAC limit of 1.

- Separate wireless SSID: SmartHUB-Gaming (WPA2, isolated from other SSIDs).

## 9.3 Internet Café zone (VLAN 30)

- Members must be able to access the shared network printer.

- Members must be able to upload and download files from the FTP server using FTP (TCP port 21)

- All other access to VLAN 50 (servers) must be denied by ACL.

- Internet access is permitted via NAT/PAT.

- Wireless SSID: SmartHUB-Cowork (WPA2-Enterprise preferred; WPA2-PSK acceptable).

- DHCP lease time of 8 hours to accommodate full working-day sessions.

## 9.4 Staff and administration zone (VLAN 40)

- Staff PCs and point-of-sale terminals must have statically assigned IP addresses.

- Full access to all internal VLANs is required only for the Amin pc.

- Network Admin must be able to SSH into all network devices on VLAN 99.

- Access all server resources.

- The billing and session management system (VLAN 50) must be accessible from this VLAN.

- CCTV and NVR management interfaces must be reachable from the manager's workstation only.

## 9.5 Server / DMZ zone (VLAN 50)

All servers are connected to a dedicated access switch (SW-5) which uplinks exclusively to both MLS distribution switches. SW-5 carries VLAN 50 traffic only and has no end-user devices connected to it. This dedicated switch ensures server traffic is isolated at the physical layer, prevents congestion from end-user zones affecting server performance, and reduces the attack surface by ensuring no guest or staff device shares a switch with the billing system, DNS server, or CCTV NVR.

- DHCP server: centralized pool management for VLANs 10, 20, and 30 via Ip helper-address.

- DNS server: resolves internal hostnames; forwards external queries to ISP DNS (8.8.8.8).

- HTTP server: hosts the captive portal landing page on port 80.

- FTP / file server provides shared storage for internet café customers on port 21.

- CCTV NVR: accessible from VLAN 40 only; no inbound connections from guest VLANs.

- No server in this zone may initiate outbound connections to VLAN 10 or VLAN 20.

## 9.6 Management zone (VLAN 99)

- Every managed switch and the core router must have an SVI or sub-interface on VLAN 99.

- SSH version 2 must be the only permitted management protocol (Telnet must be disabled).

- Access to VLAN 99 must be restricted to VLAN 40 (Network Admin) source addresses only via ACL.

- NTP must be configured on all devices – the router acts as the NTP master.

- No data traffic from any other VLAN is permitted to traverse VLAN 99.

# 10. Technologies to be Implemented

The following table lists all networking technologies required for this project, their purpose within the SmartHUB infrastructure, and the intended implementation approach.

| Technology | Purpose | Implementation approach |
| --- | --- | --- |
| VLANs (802.1Q) | Zone segmentation – 6 VLANs | Trunk all inter-switch links; access ports per zone |
| Inter-VLAN routing | Inter-vlan routing via SVI | SVI per VLAN |
| DHCP | Centralized server on VLAN 50 | ip helper-address on each SVI pointing to VLAN 50 server |
| DNS | Internal resolver forwarding to ISP | Resolve café internal hostnames; forward public queries |
| NAT / PAT | All guest VLANs share one public IP | Overload on WAN interface; permit VLANs 10, 20, 30, 40 |
| Extended ACLs | Zone isolation and service restriction | Named ACLs applied inbound on SVIs |
| STP / RSTP | Loop prevention across all access switches | Root bridge = Layer 3 switch; portfast on access ports |
| EtherChannel (PAgP) | Aggregated uplink – L3 switch to router | Two Gi links bundled; provides redundancy and bandwidth |
| Port security | MAC address lockdown on guest ports | Sticky MAC, max 1 per port, violation shutdown |
| SSH v2 | Secure management of all network devices | Disable Telnet; RSA 2048-bit keys; VTY ACL from VLAN 40 |
| NTP | Clock synchronization across all devices | Router as NTP master; all switches sync to router |
| HTTP server | Captive portal landing page for guests | PT HTTP server in VLAN 50; guests redirected on first request |
| Dynamic routing | Dynamic routes to LAN | OSPF area 0 |
| Static routing | Default route to simulated ISP | ip route 0.0.0.0 0.0.0.0 [ISP gateway] |
| Management VLAN 99 | Out-of-band device management | SVIs on VLAN 99 for SSH access; no data traffic |

# 11. Security Requirements & ACL Policy

## 11.1 General security principles

- Inter-VLAN traffic is routed by the distribution multilayer switches and inspected using ACLs before being forwarded.

- Default denies traffic not explicitly permitted by an ACL rule must be dropped.

- All ACLs must be named extended ACLs applied inbound on the relevant SVI.

- ACL change records must be documented with the date, author, and business justification.

- Port security violation action: shutdown (with manual recovery by staff).

- All management sessions must use SSH v2 with RSA keys of minimum 1024 bits.

## 11.2 Additional security controls

- BPDU Guard must be enabled on all access switch ports to prevent STP topology attacks.

- Dynamic ARP Inspection (DAI) must be configured on guest VLANs (10 and 20).

- Unused switch ports must be shut down and assigned to an unused VLAN (e.g. VLAN 888).

# 12. Wireless Requirements

| SSID name | Mapped VLAN | Security | Notes |
| --- | --- | --- | --- |
| SmartHUB-Public | VLAN 10 | WPA2-PSK | Shared password displayed at reception; captive portal enforced |
| SmartHUB-Gaming | VLAN 20 | WPA2-PSK | Separate password for gaming zone; client isolation enabled |
| SmartHUB-Cafe | VLAN 30 | WPA2-PSK | Café customer password; longer session lease supported |
| SmartHUB-Staff | VLAN 40 | WPA2-PSK | Internal only; SSID broadcast disabled |

# 13. Project Delivery & Milestones

The project is structured in five phases. Each phase must be verified with Cisco Packet Tracer simulation evidence (show commands and ping/traceroute outputs) before the next phase begins.

| Phase | Milestone | Timeline | Deliverable / evidence |
| --- | --- | --- | --- |
| Phase 1 | Layer 2 foundation | Week 1–2 | VLANs, trunking, STP, EtherChannel, port security – with show command screenshots |
| Phase 2 | Layer 3 & services | Week 3 | Inter-VLAN routing, DHCP, DNS, static routing – ping matrix across all VLANs |
| Phase 3 | Internet & NAT | Week 4 | NAT/PAT, captive portal HTTP server, default route – guest browsing demonstration |
| Phase 4 | Security hardening | Week 5 | Extended ACLs (all 8 rules), SSH, port security – before/after access verification |
| Phase 5 | QoS & finalisation | Week 6 | DSCP marking, NTP, full topology walkthrough, LinkedIn documentation post |

# 14. Acceptance Criteria

The network infrastructure will be considered accepted when all the following conditions are met:

- All six VLANs are correctly configured and end devices receive IP addresses via DHCP.

- A device in VLAN 10,20,and 30 can reach the internet but cannot ping any device in VLAN 40 or VLAN 50 (except DNS).

- VLAN 10,20, and 30 cannot communicate with one other.

- A device in VLAN 30 (Internet Café) can reach the file server but cannot reach the billing server.

- A Network Admin device in VLAN 40 can reach all other VLANs and SSH into any network device.

- No device outside VLAN 40 can SSH into any network device.

- The captive portal HTTP page loads correctly in a guest browser (simulated via PT HTTP client).

- NTP synchronisation is confirmed on all switches (show ntp status).

- EtherChannel is active and load-balancing between bundled interfaces.

- STP root bridge is confirmed as the Layer 3 switch on all VLANs.

- Port security violation is triggered when a second MAC address is connected to a guest port.

# 15.  Appendix – Reference Standards

- IEEE 802.1Q – VLAN tagging and trunking

- IEEE 802.3ad / 802.1AX – Link aggregation (EtherChannel / LACP)

- IEEE 802.1D / 802.1w – Spanning Tree Protocol / Rapid STP

- RFC 1918 – Private IP addressing (IANA)

- RFC 2827 – Network ingress filtering (BCP 38)

- RFC 4271 – SSH Protocol Architecture

- Cisco IOS Configuration Guide – Cisco Systems, Inc.

End of document